PART VI — INSTRUMENT TRAYS ORTHOPAEDIC TRAYS Complete, illustrated examination notes General · Gynaecologic & Obstetric · Genitourinary · Thoracic Cardiovascular · Orthopaedic · Neurologic · ENT · Ophthalmic · Paediatric Legal (8.5 × 14 in) · 1.27 cm margins · monochrome print-ready Every figure is an original line diagram, drawn for clarity in black and white

Table of Contents

The page numbers are live fields, shown as “—” until they are filled in.  **To update them: press Ctrl+A then F9** (on a Mac, Cmd+A then Fn+F9) — or right-click any number and choose “Update Field”. Word normally offers to do this when the file opens. Once updated, each number is also a link: Ctrl+click it to jump to that tray.

| Tray / Chapter | Page |
| --- | --- |
| ORTHOPAEDIC TRAYS | — |
| 6.1 Basic Orthopaedic Procedures Tray | — |
| 6.2 Minor Orthopaedic Procedures Tray | — |
| 6.3 Hip Replacement Tray | — |
| 6.4 Knee or Ankle Arthroscopy Tray | — |

How to Read These Notes

Every tray chapter is built to the same pattern, so you can find the same kind of information in the same place each time. Read the lead paragraph and the figure first to fix the idea, then work through the contents table.

| Lead paragraph | What the tray is for and the single idea that shapes it. If you remember nothing else from a chapter, remember this. |
| --- | --- |
| Used for / includes | The procedures the tray serves — useful for 'name three indications' questions. |
| Figure | An original line diagram of the tray's signature instruments — the items that identify it. Diagrams are schematic: they show the identifying feature (jaw pattern, bevel direction, footplate, curve) rather than a photographic likeness. |
| Callout box | The one concept, distinction or safety rule that examiners return to for that tray. |
| Contents of the tray | The full instrument list, grouped by function — cutting, grasping, clamping, retracting, suturing, suctioning — with quantities and the reason each item is present. Answer 'enumerate the contents' questions using these groups as headings. |
| Additional supplies | Position, draping, structures at risk and procedure-specific notes. |
| Comparison table | Where two trays or two instruments are easily confused, they are set side by side. |
| Examination pointers | The specific facts most often awarded marks — learn these last, as revision. |
| Common mistakes | Errors that lose marks, marked with ✗. |
| Likely examination questions | Questions with model answers written at the length an examiner expects. |
| A note on quantities | Instrument names, functions and groupings are standard. The numbers ("6 Allis clamps") vary between institutions — please cross-check them against your prescribed textbook. |
| Printing | Legal paper (8.5 × 14 in), 1.27 cm margins, monochrome throughout. No information depends on colour, so a black-and-white printer loses nothing. |

PART VI # Orthopaedic Trays Basic orthopaedic · Minor orthopaedic · Hip replacement · Knee or ankle arthroscopy

Orthopaedic trays are unlike all others because the target tissue is bone. Bone must be cut, shaped, held, drilled and fixed, so the trays carry osteotomes, chisels, gouges, saws, rasps, rongeurs, curettes, mallets, bone-holding clamps and powered instruments. Two further features are constant: intra-operative imaging and an uncompromising attitude to asepsis, because bone and implant infection is a catastrophe.

## 6.1   Basic Orthopaedic Procedures Tray

Also called:  Basic bone tray · fracture fixation tray · general orthopaedic set

| The basic orthopaedic tray is the reference bone set. It contains the soft-tissue instruments of a major tray plus a complete range of bone-cutting, bone-shaping and bone-holding instruments and the means to drill and fix. Every other orthopaedic tray in this Part is this set plus procedure-specific implants and jigs. Used for / includes ▪ Open reduction and internal fixation of long-bone fractures ▪ Plating, screw fixation and intramedullary nailing ▪ Osteotomy and corrective bone surgery ▪ Bone grafting — harvest and application ▪ Debridement and sequestrectomy for osteomyelitis ▪ Amputation; excision of bone tumours ▪ Arthrodesis of a major joint |  | Fig 6.1 — The bone-working core of every orthopaedic tray |
| --- | --- | --- |

The three bevels ▪ Osteotome — bevelled on both faces; cuts and splits. ▪ Chisel — bevelled on one face; cuts a flat surface. ▪ Gouge — U-shaped trough; scoops cancellous bone. ▪ This is the most frequently asked orthopaedic distinction — learn it verbatim.

### Contents of the tray

| Qty | Instrument | Key feature / rationale |
| --- | --- | --- |
| BASE SET |  |  |
| 1 | Major procedures tray | soft-tissue exposure and closure |
| BONE CUTTING AND SHAPING |  |  |
| 6 | Osteotomes, graded widths | bevelled on both faces — cuts through cancellous bone, splits and shapes |
| 4 | Chisels, graded widths | bevelled on one face only — cuts a flat surface, raises a cortical flake |
| 4 | Gouges, graded widths | U-shaped (troughed) blade — scoops cancellous bone and takes bone graft |
| 2 | Mallet, metal and nylon-faced | never a hammer — drives osteotomes and impactors |
| 2 | Bone rongeur (single- and double-action) | nibbles away bone in small bites |
| 2 | Bone cutting forceps (Liston / Horsley) | divides small bones |
| 4 | Bone curettes, straight and angled | removes cancellous bone, granulation and tumour |
| 2 | Bone rasp / file | smooths a sharp bone edge — prevents soft-tissue injury |
| 1 | Gigli saw with two handles | flexible wire saw for a through-and-through cut |
| 1 | Amputation / hand saw | manual back-up for straight division of a long bone |
| 1 | Oscillating power saw with blades | the modern method of straight bone division |
| BONE HOLDING AND REDUCTION |  |  |
| 4 | Bone-holding forceps (Lane / Fergusson) | signature item — heavy ratcheted toothed clamps gripping the shaft |
| 2 | Bone-holding forceps, self-centring (Lowman) | holds a plate to the bone |
| 2 | Reduction forceps, pointed (Verbrugge) | holds fragments reduced |
| 2 | Bone levers / spikes (Trethowan, Bristow) | levers fragments into position |
| 2 | Periosteal elevator (Farabeuf / Cobb) | raises periosteum before cutting |
| 1 | Bone hook | manipulates fragments |
| 2 | Bone clamps, serrated | hold two fragments in reduction while a plate is applied |
| DRILLING AND FIXATION |  |  |
| 1 | Power drill with chuck, guard and keys | battery or air-driven; sterile motor |
| 1 | Hand drill (Hudson brace) with drill bits | back-up if power fails |
| — | Drill bits, graded diameters | matched to screw core diameter; a blunt bit burns bone |
| 1 | Drill guide (neutral and load) and sleeves | directs the bit accurately |
| 1 | Depth gauge | signature item — measures the hole so the correct screw length is chosen |
| 1 | Tap and tap handle | cuts a thread for a cortical screw |
| 1 | Countersink | seats the screw head flush |
| 2 | Screwdrivers — hexagonal, cruciate, self-retaining | self-retaining pattern stops the screw dropping into the wound |
| — | Bone plates — assorted patterns and lengths | DCP, locking, reconstruction |
| — | Screws — cortical, cancellous, locking, assorted lengths | with an organised screw rack |
| 1 | Plate bender and plate-holding forceps | contours the plate to the bone |
| — | Kirschner wires, Steinmann pins and cerclage wire | with a wire passer, tensioner and cutter |
| — | Intramedullary nails with reamers, guidewires and jigs | if nailing is planned |
| 1 | External fixator set | for open or contaminated fractures |
| IRRIGATION, IMAGING AND ACCESSORY |  |  |
| 1 | Pulsatile lavage / irrigation system with warm saline | bone debris and marrow must be washed out |
| 1 | Bone graft harvesting set with a graft container | usually iliac crest |
| — | Bone substitute / allograft chips | if autograft is insufficient |
| — | Bone wax | cancellous bleeding |
| 1 | Image intensifier (C-arm) with sterile drape | essential — reduction and implant position confirmed |
| — | Lead aprons and thyroid shields for all staff | fluoroscopy is used repeatedly; protection is mandatory |
| 1 | Pneumatic tourniquet with cuff and pressure monitor | provides a bloodless field; time must be recorded |
| 1 | Suction (Yankauer) with wide-bore tubing | bone debris blocks fine tips |
| — | Plaster of Paris / splinting materials | immobilisation after fixation |
| — | Closed suction drain | bone bleeds after fixation, and a haematoma invites infection |
| — | Specimen and microbiology containers | bone and tissue for culture in infection |

#### Additional supplies and accessories

Position — Depends on the bone — supine, lateral or prone; a fracture table or traction table is used for the femur and hip.

Tourniquet — Applied over padding, inflated to about 100 mmHg above systolic for the arm and 150 mmHg above systolic for the thigh; safe duration approximately 90–120 minutes, and the time of inflation is recorded and announced.

Asepsis — Orthopaedic implant surgery uses the strictest asepsis — laminar-flow theatre, double gloving, impervious gowns, minimal traffic, and antibiotic prophylaxis given within 60 minutes of incision.

| Instrument | Blade form | Action | Typical use |
| --- | --- | --- | --- |
| Osteotome | Bevelled on both sides | Splits along a line | Cutting cancellous bone, osteotomy |
| Chisel | Bevelled on one side only | Cuts a flat plane | Raising a cortical flake, flattening |
| Gouge | U-shaped trough (concave) | Scoops a channel | Harvesting cancellous bone graft |
| Rongeur | Cup-shaped biting jaws | Nibbles small bites | Trimming bone edges, laminectomy |
| Curette | Spoon / loop | Scrapes | Removing granulation, tumour, cancellous bone |
| Rasp / file | Abrasive cross-hatched face | Abrades | Smoothing a sharp bone edge |

Table 6.1 — Bone-cutting instruments distinguished by blade form

Examination pointers ▪ Osteotome (both bevels) / chisel (one bevel) / gouge (U-shaped) — this trio is almost guaranteed to be asked. ▪ A mallet, never a hammer, is used with these instruments. ▪ Depth gauge → correct screw length is a small but commonly rewarded chain of reasoning. ▪ Image intensifier with lead protection and a pneumatic tourniquet with recorded time are part of tray preparation. ▪ Pulsatile lavage to remove bone debris and bone wax for cancellous bleeding. ▪ Mention laminar flow and strict asepsis because implant infection is devastating. ▪ A hand drill (Hudson brace) is kept as back-up if power fails.

#### Likely examination questions

Q1.  List the bone instruments in a basic orthopaedic tray and state the use of each.

A.  Osteotomes (bevelled both sides — split and cut cancellous bone); chisels (bevelled one side — cut a flat surface); gouges (U-shaped trough — scoop cancellous graft); mallet (drives them); rongeurs (nibble bone in small bites); bone cutting forceps (divide small bones); bone curettes (scrape granulation and cancellous bone); rasp/file (smooth sharp edges); Gigli and oscillating power saws (divide bone); bone-holding forceps such as Lane and Lowman and pointed reduction forceps (grip and reduce fragments); periosteal elevators and bone levers; and the drilling and fixation group — power drill with bits, drill guide, depth gauge, tap, countersink, screwdrivers, plates, screws, K-wires and cerclage wire.

Q2.  Differentiate an osteotome, a chisel and a gouge.

A.  An osteotome is bevelled on both faces and is used to split and cut cancellous bone along a line; a chisel is bevelled on one face only and cuts a flat surface or raises a cortical flake; a gouge has a U-shaped troughed blade and scoops out cancellous bone, chiefly for harvesting graft.

## 6.2   Minor Orthopaedic Procedures Tray

Also called:  Small bone tray · hand and foot surgery set

| The minor orthopaedic tray is the small-bone equivalent of the basic set: the same instruments in fine, short patterns for the bones of the hand, wrist, foot and ankle, together with the fine soft-tissue instruments those regions demand. Used for / includes ▪ Carpal tunnel and ulnar nerve decompression ▪ Trigger finger and de Quervain's release ▪ Excision of a ganglion; Dupuytren's fasciectomy ▪ Fixation of metacarpal, phalangeal and metatarsal fractures ▪ Hallux valgus correction; hammer-toe correction ▪ Removal of exostoses; K-wire fixation; tendon repair ▪ Removal of superficial orthopaedic implants |  | Fig 6.2 — The same principles in miniature |
| --- | --- | --- |

Why bipolar diathermy ▪ In digits the nerves and vessels lie within millimetres of the operative field. ▪ Bipolar current passes only between the two tips, so there is no spread to adjacent structures. ▪ Monopolar diathermy risks thermal nerve injury and, in a digit, tissue loss.

### Contents of the tray

| Qty | Instrument | Key feature / rationale |
| --- | --- | --- |
| BASE SET |  |  |
| 1 | Basic / minor procedures tray | see 1.2 |
| SMALL BONE INSTRUMENTS |  |  |
| 4 | Osteotomes, small (2–8 mm) | bevelled both sides; splits small cancellous bone |
| 2 | Chisels, small | bevelled one side; cuts a flat surface on a phalanx or metatarsal |
| 2 | Gouges, small | u-shaped trough for scooping small volumes of graft |
| 1 | Mallet, small | light controlled blows; a heavy mallet shatters small bones |
| 2 | Bone rongeur, fine (Luer, small) | nibbles small bites of bone from an exostosis |
| 2 | Bone nibbler / small bone cutter | divides a phalanx or metatarsal cleanly |
| 4 | Bone curettes, small and angled | scrapes out a cyst or granulation from a small bone |
| 2 | Small bone rasp / file | smooths a cut edge so it cannot irritate tendon or skin |
| 1 | Small oscillating saw with fine blades | precise osteotomy in hallux valgus correction |
| 2 | Small bone-holding forceps | grips a metacarpal or metatarsal shaft during fixation |
| 2 | Fine reduction forceps | pointed tips hold small fragments reduced while a wire is passed |
| 2 | Small periosteal elevator (Freer) | lifts periosteum from a small bone without stripping it widely |
| 1 | Small bone hook and elevator | manipulates a small fragment into position |
| FINE SOFT-TISSUE INSTRUMENTS |  |  |
| 2 | Fine (Adson / Gillies) toothed forceps | fine teeth for skin of the hand and foot |
| 2 | Fine non-toothed forceps | handles tendon, nerve and vessel atraumatically |
| 2 | Iris and tenotomy scissors, fine | very fine division of tendon sheath and fascia |
| 2 | Fine curved Metzenbaum scissors | blunt dissection along tendon and neurovascular planes |
| 8 | Mosquito artery forceps, fine | the principal haemostat in hand and foot surgery |
| 2 | Skin hooks, single and double | atraumatic retraction of thin dorsal skin |
| 1 | Small self-retaining (Weitlaner) retractor | holds a short incision open, freeing the assistant |
| 2 | Small Langenbeck / Senn retractors | hand-held retraction in a shallow field |
| 1 | Nerve retractor and nerve hook | carpal tunnel and nerve decompression |
| 1 | Tendon passer / tendon hook | tendon repair and transfer |
| 1 | Fine needle holder | drives 4-0 to 6-0 needles for tendon and skin repair |
| — | Fine suture — absorbable and monofilament | absorbable to deep layers, monofilament to skin |
| FIXATION FOR SMALL BONES |  |  |
| — | Kirschner wires, fine (0.8–1.6 mm) | with a K-wire driver, bender and cutter |
| — | Mini-fragment plates and screws (1.5–2.7 mm) | with the matching mini instrument set |
| 1 | Mini drill / power driver with fine bits | drives K-wires and 1.5-2.7 mm screws |
| 1 | Mini depth gauge and screwdriver | measures a 10-14 mm hole and inserts the matching screw |
| — | Interosseous / cerclage wire, fine | simple fixation where a plate would be too bulky |
| — | Bone graft substitute, small volume | fills a small defect where autograft is not justified |
| ACCESSORY |  |  |
| 1 | Pneumatic tourniquet — arm or ankle cuff | essential for a bloodless field in hand and foot surgery |
| 1 | Exsanguinating (Rhys-Davies) roll or Esmarch bandage | empties the limb of blood before the tourniquet is inflated |
| 1 | Image intensifier (mini C-arm) with drape | small-field fluoroscopy |
| — | Lead protection | mini C-arm still emits radiation; aprons are worn |
| 1 | Hand table / arm-board | stable supported surface at the correct working height |
| 1 | Fine suction (Frazier) | small vented tip that will not draw in a digital nerve |
| 1 | Bipolar diathermy | bipolar preferred near digital nerves and vessels |
| — | Loupes / magnification | digital nerves and vessels are 1-2 mm across |
| — | Splint / plaster materials | immobilises the repair immediately at the end of surgery |

Examination pointers ▪ Define this tray as the basic orthopaedic tray in fine short patterns plus fine soft-tissue instruments. ▪ The pneumatic tourniquet with an exsanguinating roll is essential — hand and foot surgery demands a bloodless field. ▪ K-wires and mini-fragment plates/screws (1.5–2.7 mm) are the fixation of small bones. ▪ Bipolar diathermy near digital nerves — explain why. ▪ A hand table and mini C-arm are the practical additions. ▪ Include the nerve retractor and nerve hook for decompression procedures.

#### Likely examination questions

Q1.  How does a minor orthopaedic tray differ from a basic orthopaedic tray?

A.  It contains the same categories of bone instrument — osteotomes, chisels, gouges, mallet, rongeurs, curettes, rasps, saws, bone-holding and reduction forceps and periosteal elevators — but in fine, short patterns suited to the small bones of the hand, wrist, foot and ankle. It adds fine soft-tissue instruments (Adson and Gillies forceps, iris and tenotomy scissors, skin hooks, nerve retractor and hook, tendon passer), mini-fragment fixation (K-wires, 1.5–2.7 mm plates and screws with a mini drill and depth gauge), an arm or ankle pneumatic tourniquet with an exsanguinating roll, a hand table, mini C-arm, fine suction, bipolar diathermy and magnification.

## 6.3   Hip Replacement Tray

Also called:  Total hip arthroplasty (THA) tray · hip prosthesis set

| Total hip replacement removes the femoral head and resurfaces the acetabulum, replacing both with a prosthesis. The tray is a basic orthopaedic set plus three additional systems: instruments to dislocate and resect the femoral head, a matched set of reamers, broaches, trials and impactors for the specific implant, and the implant itself with cement or cementless fixation. Asepsis is at its most rigorous. Used for / includes ▪ Osteoarthritis of the hip — the commonest indication ▪ Rheumatoid and other inflammatory arthritis ▪ Avascular necrosis of the femoral head ▪ Displaced intracapsular neck-of-femur fracture in the elderly ▪ Failed hemiarthroplasty or previous surgery — revision arthroplasty ▪ Post-traumatic arthritis; hip dysplasia |  | Fig 6.3 — Resect, prepare, trial, implant, impact |
| --- | --- | --- |

Sizes must be complete ▪ Trials, reamers, broaches and implants come as a matched, manufacturer-specific system. ▪ The whole size range must be present and checked before the skin incision — a missing size stops the operation with the hip dislocated. ▪ Implant lot numbers are recorded for traceability.

### Contents of the tray

| Qty | Instrument | Key feature / rationale |
| --- | --- | --- |
| BASE SET |  |  |
| 1 | Complete basic orthopaedic procedures tray | see 6.1 |
| 1 | Major procedures tray | deep soft-tissue exposure |
| EXPOSURE AND DISLOCATION |  |  |
| 2 | Hohmann retractors, assorted | signature item — curved, spiked-tip levers retracting around the femoral neck and acetabulum |
| 2 | Charnley / Cobra self-retaining hip retractor | maintains the deep exposure |
| 2 | Deep Deaver and Langenbeck retractors | supplementary hand-held retraction around the acetabulum |
| 1 | Bone hook | dislocating the femoral head |
| 1 | Corkscrew / femoral head extractor | signature item — screws into the head to remove it |
| 2 | Large osteotomes and periosteal elevators | capsulotomy and soft-tissue release |
| 1 | Femoral neck osteotomy guide and oscillating saw | resects the neck at the planned level |
| 1 | Femoral elevator / skid | levers the femur during exposure |
| ACETABULAR PREPARATION |  |  |
| 1 | Acetabular reamers, graded sizes, with a T-handle or power reamer | signature item — hemispherical reamers prepare the socket |
| 2 | Acetabular rim rongeur and curettes | clears osteophytes and residual cartilage |
| 1 | Acetabular trial cups and sizers | confirms fit before the definitive implant |
| 1 | Cup introducer / positioner with alignment guide | sets abduction and anteversion angles |
| 1 | Cup impactor and mallet | seats the acetabular cup fully with controlled blows |
| 1 | Liner inserter and liner impactor | locks the bearing liner into the metal shell |
| — | Acetabular screws with drill and screwdriver | supplementary cup fixation |
| 1 | Pulsatile lavage | cleans the bone bed before cementing |
| FEMORAL PREPARATION |  |  |
| 1 | Canal finder / box osteotome and canal reamers | opens and sizes the medullary canal |
| 1 | Femoral broaches / rasps, graded sizes | signature item — shape the canal to the stem geometry |
| 1 | Canal brush and canal plug / cement restrictor | contains the cement distally |
| 1 | Femoral trial stems, heads and neck lengths | trial reduction assesses leg length, offset and stability before the real implant |
| 1 | Stem inserter / introducer and impactor | drives the femoral stem along the prepared canal axis |
| 1 | Head impactor | seats the femoral head on the trunnion; the taper must be clean and dry |
| 1 | Calcar planer / neck trimmer | levels the femoral neck cut so the stem collar sits flush |
| 1 | Stem and head extractor | removal in revision surgery |
| THE IMPLANT AND ITS FIXATION |  |  |
| — | Acetabular cup, liner, femoral stem and head — full size range present and checked | the complete range must be confirmed in theatre before the incision |
| — | Bone cement (PMMA) with mixing bowl, spatula, vacuum mixer and cement gun | for cemented fixation |
| — | Antibiotic-loaded cement | where indicated |
| — | Bone graft / allograft chips | acetabular deficiency |
| 1 | Cement removal set — osteotomes, curettes, ultrasonic tools | revision surgery |
| — | Trochanteric wires / cables with tensioner | if a trochanteric osteotomy is performed |
| ASEPSIS, IMAGING AND ACCESSORY |  |  |
| — | Laminar-flow theatre, body-exhaust or hood suits, double gloving, impervious gowns | periprosthetic infection is catastrophic — asepsis is part of tray preparation |
| — | Antibiotic prophylaxis within 60 minutes of incision | periprosthetic infection is catastrophic; timing determines tissue levels |
| 1 | Pulsatile lavage with warm saline and antiseptic | cleans the bone bed of debris and marrow before cementing |
| 1 | Image intensifier with drape and lead protection | component position and leg length |
| 1 | Leg-length measuring device / callipers | leg-length discrepancy is a common cause of dissatisfaction and litigation |
| 1 | Wide-bore suction with several tips | bone debris and cement |
| — | Cross-matched blood and a cell saver | blood loss is significant |
| — | Closed suction drain | decompresses the deep dead space after arthroplasty |
| — | Specimen and microbiology containers | multiple tissue samples in revision surgery to identify infection |
| — | Abduction pillow / brace | prevents early dislocation post-operatively |

#### Additional supplies and accessories

Position — Lateral decubitus with the operative side up, secured in a hip positioner, for the posterior and lateral approaches; supine for the direct anterior approach.

Approaches — Posterior (commonest; higher dislocation risk), direct lateral (Hardinge), and direct anterior (muscle-sparing, steeper learning curve).

Complications — Dislocation, periprosthetic infection, venous thromboembolism, leg-length discrepancy, periprosthetic fracture, sciatic or femoral nerve injury, aseptic loosening, heterotopic ossification and fat embolism during cementing.

Trial reduction — Before the definitive components are implanted, trial components are assembled and the hip reduced to check leg length, offset, range of movement and stability. This step is why a complete set of trials is on the tray.

Examination pointers ▪ Structure the answer as exposure → acetabular preparation → femoral preparation → implant and fixation → asepsis and accessory. ▪ Name Hohmann retractors and the femoral head corkscrew extractor — the two most characteristic instruments. ▪ Acetabular reamers and femoral broaches are the preparation signatures. ▪ Trial components and trial reduction — explain the purpose (leg length, offset, stability). ▪ Bone cement with vacuum mixer and cement gun, plus a cement restrictor. ▪ Laminar flow, hood suits, double gloving and antibiotic prophylaxis — infection control is examinable content. ▪ In revision surgery add a cement-removal set and multiple microbiology samples.

Common mistakes to avoid ✗ Not confirming the full implant size range before incision. ✗ Forgetting trial components and the purpose of trial reduction. ✗ Omitting cement preparation equipment. ✗ Neglecting asepsis and thromboprophylaxis, which carry marks.

#### Likely examination questions

Q1.  Describe the tray for a total hip replacement.

A.  A basic orthopaedic and major tray plus four systems. Exposure: Hohmann retractors, Charnley/Cobra self-retaining hip retractor, deep hand-held retractors, bone hook, femoral head corkscrew extractor, large osteotomes and periosteal elevators, neck osteotomy guide and oscillating saw. Acetabulum: graded hemispherical reamers, rim rongeur and curettes, trial cups and sizers, cup positioner with alignment guide, impactors, liner inserter and acetabular screws. Femur: canal finder and reamers, graded broaches/rasps, canal brush and cement restrictor, trial stems, heads and neck lengths, stem inserter, impactors, calcar planer and extractors. Implant and fixation: the full range of cups, liners, stems and heads checked in advance, bone cement with vacuum mixer and gun, graft and trochanteric wires. Plus laminar-flow asepsis with hood suits, antibiotic prophylaxis, pulsatile lavage, image intensifier, leg-length callipers, wide-bore suction, cross-matched blood and cell saver, drain and abduction pillow.

Q2.  What is trial reduction and why does it require a separate set of components?

A.  Trial components — a trial stem, head, neck length and cup or liner — are assembled and the hip reduced before the definitive implants are inserted, to verify leg length, femoral offset, range of movement and stability. A separate complete set of trials in every size is therefore required, so the fit can be optimised and the correct definitive implant selected without compromising a sterile implant.

## 6.4   Knee or Ankle Arthroscopy Tray

Also called:  Diagnostic and operative arthroscopy set

| Arthroscopy inspects and operates within a joint through small portals, using a rigid telescope and continuous fluid irrigation to distend the joint. The tray is a small open set plus a complete endoscopic system: scope and camera, an irrigation/inflow system, fine hand instruments designed to work through a portal, and a motorised shaver. Used for / includes ▪ Diagnostic arthroscopy of the knee or ankle ▪ Meniscectomy and meniscal repair ▪ Anterior cruciate ligament reconstruction ▪ Removal of a loose body; synovial biopsy and synovectomy ▪ Chondroplasty, microfracture and cartilage procedures ▪ Ankle arthroscopy for impingement, osteochondral lesions and debridement ▪ Lavage and debridement in septic arthritis |  | Fig 6.4 — Distend, visualise, work through portals |
| --- | --- | --- |

Fluid, not gas ▪ Joints are distended with saline or Ringer's solution, not with CO₂ as in laparoscopy. ▪ So inflow, outflow and fluid balance are central to the tray. ▪ Excess extravasation can cause swelling and, rarely, compartment syndrome — monitor the volumes.

### Contents of the tray

| Qty | Instrument | Key feature / rationale |
| --- | --- | --- |
| BASE SET |  |  |
| 1 | Basic / minor procedures tray | portals and closure |
| 1 | Basic orthopaedic tray available | for conversion to open surgery |
| THE ARTHROSCOPIC SYSTEM |  |  |
| 1 | Arthroscope, 4 mm 30° (knee) and 2.7 mm (ankle) | signature item — 70° available for difficult corners |
| 1 | Arthroscope sheath and blunt obturator | blunt, to avoid scuffing the cartilage on entry |
| 1 | Fibre-optic light cable and light source | cold light; the joint is otherwise completely dark |
| 1 | Camera head, coupler, control unit and monitor | white-balanced before insertion; the operation is done on the screen |
| 1 | Image recorder / printer | documents findings for the record and for the patient |
| — | Sterile camera and cable drapes | keeps the non-sterile camera and light cable out of the field |
| 1 | Anti-fog solution and warm saline for the lens | a fogged lens stops the operation; warming prevents condensation |
| IRRIGATION — THE DEFINING REQUIREMENT |  |  |
| 1 | Arthroscopy fluid pump or gravity inflow set with warm normal saline / Ringer's | signature requirement — the joint must be distended with fluid to create a working space |
| 1 | Inflow and outflow cannulae with taps | separate circuits maintain distension and clear debris |
| — | Several litres of irrigation fluid | fluid balance is monitored — extravasation causes swelling and, rarely, compartment syndrome |
| 1 | Suction tubing and collection system | measured outflow, so fluid balance can be tracked |
| PORTAL CREATION AND JOINT ACCESS |  |  |
| 1 | Knife handle with No.11 blade | portal skin incisions |
| 2 | Cannulae with blunt and sharp obturators | establishes each portal |
| 1 | Switching stick / exchange rod | moves the scope between portals |
| 1 | Blunt trocar and joint distractor | blunt tip protects cartilage; distraction opens a tight joint |
| 1 | Ankle / joint distractor or traction device | opens the tight ankle joint |
| 1 | Leg holder or lateral post | stabilises the knee and allows valgus/varus stress |
| HAND INSTRUMENTS (LONG, FINE, PORTAL-SIZED) |  |  |
| 1 | Arthroscopic probe / hook | the first instrument used — palpates and assesses structures |
| 2 | Arthroscopic punch / basket forceps, straight and angled | excises meniscal tissue |
| 2 | Arthroscopic grasping forceps | removes loose bodies and fragments |
| 2 | Arthroscopic scissors | divides a meniscal flap or adhesion under vision |
| 1 | Motorised shaver / burr with assorted blades | signature item — resects synovium, cartilage and bone under suction |
| 1 | Radiofrequency / electrothermal probe | ablation and haemostasis |
| 1 | Curettes and rasps, arthroscopic | prepares a cartilage defect to a stable rim |
| 1 | Microfracture awls | cartilage stimulation |
| 1 | Meniscal repair set | all-inside implants, sutures and suture passers |
| 1 | ACL reconstruction set | graft harvest instruments, aiming guides, reamers, guidewires, graft tensioner and fixation devices (screws, buttons) |
| ACCESSORY |  |  |
| 1 | Pneumatic tourniquet with cuff | improves the view if bleeding obscures it |
| 1 | Image intensifier | available for ACL tunnel placement |
| — | Local anaesthetic with adrenaline | portal infiltration; adrenaline reduces bleeding |
| — | Fine suture or adhesive strips | portal closure |
| — | Compressive dressing and cryotherapy device | limits haemarthrosis and swelling after portal closure |
| — | Specimen containers | synovium, cartilage, fluid for microscopy and culture |
| 1 | Wide-bore suction | clears resected tissue and irrigation fluid quickly |

#### Additional supplies and accessories

Position — Knee — supine with the knee over a break in the table or in a leg holder, tourniquet high on the thigh. Ankle — supine with a bolster under the calf, with or without a distraction device.

Portals — Knee: anterolateral (viewing) and anteromedial (working), with superomedial for inflow. Ankle: anteromedial and anterolateral, avoiding the superficial peroneal nerve and dorsalis pedis.

Complications — Haemarthrosis, infection (septic arthritis), iatrogenic cartilage scuffing on entry, neurovascular injury at the portals (saphenous nerve, superficial peroneal nerve), fluid extravasation and compartment syndrome, thromboembolism, and instrument breakage within the joint.

Broken instrument — A fragment of a fine arthroscopic instrument inside the joint is a retained item and must be retrieved; instruments are inspected before and after use.

|  | Laparoscopy | Arthroscopy |
| --- | --- | --- |
| Distending medium | CO₂ gas, 12–15 mmHg | Saline / Ringer's fluid under pump or gravity |
| Scope | 10 mm 0°/30° laparoscope | 4 mm 30° (knee) / 2.7 mm (ankle) |
| Obturator | Sharp or blunt trocar | Blunt — protects cartilage |
| Power instrument | Ultrasonic / bipolar devices | Motorised shaver / burr |
| Main fluid hazard | Gas embolism, hypercapnia | Extravasation, compartment syndrome |
| Conversion set | Open laparotomy tray | Open orthopaedic tray |

Table 6.2 — Endoscopic principles compared: the distending medium changes everything

Examination pointers ▪ The defining requirement is fluid irrigation to distend the joint — contrast this with CO₂ in laparoscopy. ▪ The obturator is blunt to avoid scuffing articular cartilage — a favourite detail. ▪ The motorised shaver/burr is the signature working instrument. ▪ The probe is the first instrument used for systematic assessment. ▪ Mention fluid balance monitoring and compartment syndrome as the fluid-related complication. ▪ Tourniquet, leg holder and a joint distractor for the ankle are the positioning aids. ▪ Keep an open orthopaedic tray available for conversion, and note that a broken instrument fragment is a retained item.

#### Likely examination questions

Q1.  List the contents of a knee arthroscopy tray.

A.  A minor tray for portals plus: a 4 mm 30° arthroscope (2.7 mm for ankle, 70° available) with sheath and blunt obturator, light cable and source, camera, monitor and recorder with anti-fog; an irrigation system — fluid pump or gravity set with warm saline, inflow and outflow cannulae and several litres of fluid with balance monitoring; portal instruments — No.11 blade, cannulae with obturators, switching stick, leg holder or lateral post and an ankle distractor; hand instruments — probe/hook, punch and basket forceps, graspers, scissors, curettes and rasps, motorised shaver and burr, radiofrequency probe, microfracture awls, meniscal repair set and an ACL reconstruction set with guides, reamers and fixation; plus tourniquet, image intensifier, local anaesthetic with adrenaline, portal closure materials, compressive dressing, specimen containers and an open orthopaedic tray for conversion.

Q2.  Why is the arthroscope introduced with a blunt obturator?

A.  A sharp trocar would scuff or gouge the articular cartilage on entry, causing permanent iatrogenic chondral damage. A blunt obturator pushes the capsule and synovium aside without cutting the joint surface.